Units:

Capacitance: F =A2s4kg m2

Voltage: V=kg m2s3 A

Inductance: H=kg m2s2A2

Resistance: Ω=kg m2s3A2

[p=e-12, n=e-9, μ=e-6, m=e-3, K=e+3, M=e+6, G=e+9]

R-C Circuits:

Q=CV

τ=RC

Average: <x> = 1T0Txtdt

Note that, in a series RC circuit

<Vct> =D*VS

Where D is the decimal of time the voltage source is at Vs, and (1-D) is the complement when the voltage source is off.

Ripple voltage:

vr=DVsRC1-DT;

LRC Circuits:

V=LdIdt

Low pass filters:

- Vin + L + (R||C || Vout)

- Vin + L + (R || Vout)

- Vin + R + (C || Vout)

- Vin + L + (C || Vout)

- Vin + R + L + (C || Vout) (has resonance)

High pass filters:

- Vin + R + (L || Vout)

- Vin + C + (R || Vout)

- Vin + C + (L || Vout)

- Vin + R + C + (L || Vout)

Band pass / notch filter:

- Vin + R + (L || C || Vout) – filter to a specific band

- Vin + L + C + (R || Vout) – filter everything except a specific band

Energy:

12CV2=12LI2=ddt[I*V]

Second-order circuits:

ω0=1LC

Q=ω02α

In a parallel RLC, α=12RC. In series, α=R2L. In other circuits:

- Solve for the transfer function

- Replace jw with s

- Set coefficient for highest order s-term in the denominator to 1. Then, the denominator is equivalent to the characteristic equation: s2+2αs+ω02. Note that this is a solution to:
 d2xtdt+2αdxdt+ω02=0

- Solve for ω02α

Note that, given a transfer function of the form WHERE K IS REAL:

ks2+bs+ω02;s=ωj;imagk=0

Then the angle is: ϕ=atan2(-bω,ω02-ω2)

And the magnitude is: kω02-ω22+bω2

Mobility analog:

Power = I*V		Power = ???? (https://iesc-s2.mit.edu/6002/_static/F18/lecturenotes/Lecture16.pdf)

U is terminal velocity, p is power, m is mass, k is the spring constant, x is displacement, f is force,  dλdt=v

Mass = capicator. (p=m*u⇔q=cv

Inductor = spring (x=1kf  λ=L*I)

Resister = damper (u=1Bf⇔v=R*I)

Units: